Grenada

Project Name: Procurement of Electric Bus

Agency Name: Ministry of Finance

# Procurement of Electric Buses

REQUEST FOR QUOTATIONS

FOR THE SUPPLY OF GOODS

Request for Quotation No: MOF-EV – 01 rebid

Issued on: November 2025

Table of Contents

Section 1 	Request for Quotations

Section 2 	Instructions for Tenderers

Section 3 	List of Goods and Delivery Schedule

Section 4 	Technical Specifications of the Goods Required

Section 5 	Schedule of Items and Prices

Section 6 	Quotation Form

Section 7 	Tenderer Information Form

Section 8 	Notification of Intention to Award a Contract

Section 9 	Notification to Award a Contract

Section 10 	Contract Form

Section 1: Request for Quotations

Central Procurement Unit

Ministry Of Finance

Building 5

The Carenage, St. George’s

Date: October 24, 2025

# Request for Quotations

- The Ministry of Finance, Central Procurement Unit invites you to submit a sealed Quotation for the supply of two (2) Electric Buses in the form provided in Section 6. The buses shall be delivered to the Central Procurement Unit within six (6) months from the date of the signed contract.

- Tendering will be conducted through the request for quotations procedure specified in the in the Public Procurement and Disposal of Public Property Act 2014 (“the Act”), as amended, and the Public Procurement and Disposal of Public Property Regulations 2015 (“the Regulations”).

- Your quotation must be for all items required and specified in Sections 3 and 5: Items and Prices. The Central Procurement Unit has the right to reject any quotations:

- which do not respond to all the items specified in the Sections 3 and 5;

- where the proposed technical specifications do not meet the stated requirements of Section 4.

- The Central Procurement Unit will award the contract to the Tenderer whose quotation has been determined to be most substantially responsive to the Request for Quotation and has offered the lowest evaluated quotation.

- Quotations shall be submitted in English before November 22, 2025, via the link  https://in-tendhost.co.uk/GND/aspx/Home  and shall remain valid for the period of 120 days after the deadline for submission of quotations. Late quotations will not be accepted.

- You may submit only one quotation.

Signature of authorized person

________________________________

Chief Procurement Officer

Central Procurement Unit

Section 2: Instructions to Tenderers

- The Procuring Entity invites quotations for Two (2) Electric buses set out in Section 3.

- Only Tenderers to whom the RFQ is addressed may submit a quotation.

- The Supplier should also provide a priced schedule of consumables, spare parts and routine maintenance that are expected to be required on the basis of a normal utilization of the equipment over a period of [insert number of years].

- The buses shall be subject to a warranty period of:

  - Base Vehicle Warranty: Minimum of two (2) years or 60,000 km, whichever comes first.

  - Battery Pack Warranty: Minimum of 5 years or 15,000 km whichever comes first, covering failure and degradation below 70% State of Health (SoH)

  - Drive Unit Warranty (Motor, Inverter, Gearbox): Minimum of 5 years or 15,000 km whichever comes first.

  - Bidder must provide full warranty terms and conditions, including local support and claim procedures in Grenada.

- Quotations shall be submitted in Eastern Caribbean Dollars (EC$).

- The Tenderer shall bear all costs associated with the preparation and submission of its quotation and the Procuring Entity shall not be responsible or liable for those costs, regardless of the conduct or outcome of the tendering process.

- The Government requires compliance with its integrity policy in accordance with section section 26 of the Act.

- Your price quotation must be submitted electronically through the electronic procurement system. Bidders are requested to register, submit quotes, and address any queries, through the following portal: https://in-tendhost.co.uk/GND/aspx/Home

- Information relating to the evaluation of quotations shall not be disclosed to Tenderers or any other persons not officially concerned with the tendering process until information on the Contract Award is communicated to all Tenderers. Any effort by a Tenderer to influence the Procuring Entity in the evaluation of the tender or contract award decision may result in the rejection of its Quotation.

- In evaluating the quotations, the Procuring Entity will determine for each quotation the evaluated price by adjusting the quotation by marking any correction for any arithmetic errors as follows:

  - where there is a discrepancy between amount in figures and in words, the amount in word will govern,

  - where there is a discrepancy between the unit rate and the line item total resulting from multiplying the unit rate by the quantity, the unit rate as quoted will govern, unless in the opinion of the Procuring Entity there is an obvious misplacement of the decimal point in the unit price, in which case the line item total as quoted shall govern and the unit price shall be corrected.

- Depending on the final requirement, the Procuring Entity may increase or decrease the quantity or items list. If any such change causes an increase or decrease in the quantity or items list, the rationale for the change shall be specified in the evaluation report for the procurement.

- The Procuring Entity reserves the right to accept or reject any quotations, and to cancel the process of the competition and reject all quotations, at any time prior to the award of the contract, without thereby incurring any liability to affected Tenderers or any obligation to inform the affected Tenderers of grounds.

- Prior to the expiration of the period of validity of the quotation, the Procuring Entity shall send to each Tenderer the Notification of Intention to Award the Contract by using the form in Section 8.

- The Contract shall be awarded within 10 days commencing the day after the date the Procuring Entity has transmitted to each Tenderer the Notification of Intention to Award a Contract. The Notification of Intention to Award shall contain, at a minimum, the following information:

- the name and address of the Tenderer submitting the successful Quotation;

- the price of the successful Quotation;

- the names of all Tenderers who submitted Quotations and the prices quoted by them; and

- instructions on how to submit a complaint or request a debrief.

- Prior to the expiration of the period of tender validity and upon satisfactorily addressing any complaint that has been submitted in accordance with Part VII of the Act, the Procuring Entity shall send to the successful Tenderer(s), in writing, a Notification to Award a Contract.

- Any Tenderer has the right to complain if it has suffered or may suffer loss or damage due to a breach of a duty imposed on the Procuring Entity pursuant to Part VII of the Act.

Section 3: List of Goods and Delivery Schedule

| Item | Name and Description of Goods | Unit | Quantity | Point of Delivery | Date of Delivery (days/weeks/months) |
| --- | --- | --- | --- | --- | --- |
| 1 | Electric Buses | pcs | 2 | Ministry of Finance, Galleria Mall |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |

Signature of Supplier

Insert Name of Supplier

Section 4: Technical Specifications of the Goods Required

| Item | Name/Description | Technical Specification required | Technical Specification offered |
| --- | --- | --- | --- |
|  |  | Minimum of 90kW |  |
|  | Type: | AC Permanent synchronous motor |  |
|  | Max. Power: | 120/190 KW |  |
|  | Max. Torque: | 250/300NM |  |
|  | Max Speed | 100km/h |  |
|  | Battery type | Lithium Iron Phosphate |  |
|  | Cell Voltage | 3.2V |  |
|  | Range | Minimum 300 km |  |
|  | 2. CHARGING | Both AC and DC fast charging options (CCS2 or GBT) should be provided. |  |
|  | Cooling objects | motor, motor controller, steering motor controller, DC-DC, etc. |  |
|  | Cooling fluid types and levels | Antifreeze -25 |  |
|  | Battery Compartment | Should be completely sealed to prevent fumes and heat from entering the passenger compartment. |  |
|  |  | Should have a safety interlock switch to prevent vehicle from being engaged from driver's compartment while the battery compartment door is open |  |
|  |  | Should allow artisan to start vehicle from battery compartment |  |
|  | 3. STEERING | Power assisted right hand mounted incorporating turn signal and dimmer |  |
|  | Steering Wheel | Steering Wheel with horn button |  |
|  | 4. FRONT AXLE | Reverse Elliot: I Beam [Preferred] |  |
|  | 5.FRONT SUSPENSION |  |  |
|  | Type: | Pneumatic with 2 air bags and auxiliary internal backstop height regulating valve. |  |
|  | Shock Absorbers | Hydraulic double acting telescopic |  |
|  | 6. REAR AXLE | Banjo, fully floating. Vehicle must be able to attain a top speed of at least 90/100km/h |  |
|  | 7.REAR SUSPENSION |  |  |
|  | Air Bag | Pneumatic with height regulating valves |  |
|  | Shock Absorbers | Hydraulic double acting telescopic |  |
|  | 8. ROAD WHEELS | Fitted on tubeless tyres (9R22.5 radial steel belted or another suitable size) Hubometer to be fitted to rear wheel on same side with fuel lid |  |
|  | 9. BRAKES |  |  |
|  | Service | Full air dual circuit - Front Disc, Rear Drum |  |
|  | Parking | Disc with wear indicator (electronic or manual) |  |
|  | ABS | Anti-Lock Braking System |  |
|  | 10. ELECTRICAL |  |  |
|  | Horn | Electrical Type |  |
|  | Front lamp | High beam/low beam |  |
|  |  | Amber (turn) and white (parking) |  |
|  | Rear Lamp | Amber (turn) and red (stop/parking) |  |
|  | Back up lamp | White |  |
|  | All external indicator identification lights to be flush with bus body as far as possible] |  |  |
|  | 11. RADIO |  |  |
|  | Eight (8) Speakers | Buses to be fitted with two-way radios |  |
|  |  | Buses to be fitted with AM/FM radios and USB player |  |
|  |  | Buses to be fitted with Audio playback system |  |
|  | not more than three (3) monitors | Buses to be fitted with PA Systems |  |
|  | 12.INSTRUMENT PANEL |  |  |
|  | Lighting switches | Material – Preferably hard-plastic |  |
|  | Battery isolator switch | Located on meter panel |  |
|  | Starter switch | Located on meter panel |  |
|  | Instrument cluster | Located on dashboard |  |
|  | Indicators: | Voltmeter, Amp Meter, Water Temperature Gauge, Engine Oil Pressure Gauge, Air Pressure Parking Brake, Charging, Low Air Pressure, Low Water Level, High Water. Temperature, Hazards Warning Light or Buzzer, Tachometer (rpm/hours), Gauge for fuel showing fuel level Low Fuel Indicator [Audible & Visual] |  |
|  | 13.BODY CONSTRUCTION | Must be treated with anticorrosive substance (rust proof) |  |
|  | Length/Width/Height | Body length should be minimum.7500mm (from bumper to bumper), width should be 2350mm, height should be 3365mm |  |
|  | Frame exterior panels | Steel or aluminum panels secured by riveting, welding or any other method |  |
|  | Interior panels | Molded plastic ceiling lining |  |
|  | Fender | Sheet steel or aluminum for front and rear |  |
|  | Steps | Checkered steel or aluminum checkered plate (non-skid floor material) |  |
|  | Painting | Yellow or green/client requested |  |
|  | 14. DOOR |  |  |
|  | Entrance | Automatic folding or outward opening one piece door to be located on the left side with direct passenger access to the driver. Door release emergency valve |  |
|  | Emergency Exit | Located at the right side and fitted with fixed window. |  |
|  | Emergency Door release Lever | Emergency door release lever internal and external (For opening and closing door) |  |
|  | 15. WINDOWS AND RELATED COMPONENTS |  |  |
|  | Front Windshields | One piece windshield with 250MM tinted top |  |
|  | Rear Window | One piece rear window |  |
|  | Side Window and sky windows | Fixed upper section and moveable side sliding lower section with aluminum alloy slashes: (Glazed tempered glass). At least 2 sky windows that can be used for ventilation and for escape in emergency. |  |
|  | Destination window | Fitted above windscreen (Front) 720mm length 220mm height destination board |  |
|  | 16. FLOORING |  |  |
|  | Floor | Marine plywood flooring covered with PVC or plastic tiles |  |
|  | Accessible floor inspection hatches | Fitted with pulls and locks made from appropriate rust resistant material |  |
|  | 17. SEATING |  |  |
|  | Seating capacity | 22 seated Two (2) full seats on either side (adjustable seats) |  |
|  | Seats | Seats must not be less than 300 to 350 mm apart. All passengers seat fitted with seat belt |  |
|  | Seat Material | fabric or leather |  |
|  | Driver’s Seat | Pedestal type, adjustable up and down, forward and backward to turn 90° Left and Right (Fire Resistant) Seat must be Leatherette/ or fabric and fitted with seat belt |  |
|  | 18. AIR CONDITION | Buses to be fitted with Motor Driven Air Condition Unit with Controls. An additional quotation may be given on other systems |  |
|  | 19. ELECTRICAL EQUIPMENT |  |  |
|  | Room lamps | high intensity LED |  |
|  | Step wall lamp | Flush mounted, door actuated open and close |  |
|  | Head lamp | High beam/low beam (Halogen) |  |
|  | Back up lamps | Right rear and left rear |  |
|  | Tail and stop lamp | One (1) each on right and left, turn indicator, hazard warning and parking lamps |  |
|  | Reverse warning buzzer | Intermittent or continuous & reverse camera |  |
|  | Emergency alarm | Alarms when emergency door is open |  |
|  | Clock | Bus to be fitted with digital clock |  |
|  | Buzzer system for alighting passengers | Push button type |  |
|  | Accelerator Interlock device | There must be no acceleration when doors are open |  |
|  | Driver's Cabin | To be equipped with individual light |  |
|  | Bus stop announcer | Automatic station announcer (with speaker in English) |  |
|  | Broadcasting system | 1 |  |
|  |  | 1 |  |
|  | Monitor system | Interior, 1 |  |
|  |  | Exterior, 1 |  |
|  | External rear view mirrors | Suspended type, convex and remote controlled fitted on both side and must be easily collapsible inward to body. |  |
|  | Driver fan | 1 |  |
|  | CAN system | CAN system |  |
|  | Multimedia system | 1 |  |
|  | Vehicle traveling data recorder/Tachograph | 1 |  |
|  | Events Data Recorder (EDR) | 1 |  |
|  | Instrument display and buttons | 1 |  |
|  | Digital clock | 1 |  |
|  | Wi-Fi | 1 |  |
|  | Automatic passenger counting system | 1 |  |
|  | AVL/AVM system | 1 |  |
|  | Multiplexing system | 1 |  |
|  | Smart transportation system | 1 |  |
|  | Mobile Data Terminal (MDT) | 1 |  |
|  | 20. EXTERNAL LIGHTS | Must be flush with body as far as practical |  |
|  | 21. GROUND CLEARANCE | Clearance between lower edge of side panel and ground, minimum of 0.30m – 0.35m |  |
|  | 22. GRADABILITY | Minimum 30% or higher |  |
|  | 23. TACO GRAPH | Bus must be fitted with Taco graph or a system that can provide same information as taco graph (Taco graph reader to be provided by manufacturer) |  |
|  | 24. PARK PILOT | To incorporate system for parking bus (visual or audio) |  |
|  | 25. ACCESSORIES |  |  |
|  | Fire extinguisher | 2.27 kg dry chemical type |  |
|  | Safety hammer | For breaking window glass in cases of emergency |  |
|  | Tool Kit Special/ Diagnostic tools scanner & connector engine& G box | Standard – to include wheel spanner, sockets and power bar, Any specialist tool & equipment. i.e. screwdrivers, tips and inclusive of any diagnostic tools and software etc. |  |
|  | Options | Automatic Extinguishers, Running Recorder, Over speed Protection |  |
|  | Manuals | All information must be in English – and in both hard copy and soft copy. |  |
|  |  | Repair operation manuals for all operating systems and components |  |
|  |  | Parts manual |  |
|  |  | Parts listing to be supplied |  |
|  |  | Diagnostic tools (scanner) for chassis components, engine, transmission etc. |  |
|  |  | One (1) Garbage bin located close to driver for ticket disposal |  |
|  | Spare Parts | List of recommended spares must form part of suppliers' submission. |  |
| 28.DESIGN REQUIREMENTS |  |  |  |
| The vehicle and its components should comply with the following statutory and internationally recognized requirements: |  |  |  |
| Motor Vehicle and Road Traffic Ordinance of Grenada |  |  |  |
| British Public Service Vehicle Regulations for construction use |  |  |  |
| Public Service Vehicle (PSV) Lighting |  |  |  |
| Secondary Legislation of European Community Technical Standard with specific reference to vehicle breaking equipment and performance |  |  |  |
| Recommendations for the standardization of motor buses issued by the International Union of Public Transport [IUTP] |  |  |  |
| United States Federal Motor Vehicle Safety Standard applicable to [PSV] Buses |  |  |  |
| Where specified, the materials and components offered should meet the American Society of Automobile Engineers [SAE] standards and performance recommendations. |  |  |  |

Signature of Supplier

Insert Name of Supplier

Section 5: Schedule of Items and Prices

| Item | Name and Description the Goods | Unit | Quantity (1) | Unit Rate (EC$) Included all duties and taxes, transportation, insurance to final destination (2) | Total Amount EC$ (3) = (1) x (2) |
| --- | --- | --- | --- | --- | --- |
| 1 | Electric Buses | pcs | 2 |  |  |
|  |  |  |  | Grand Total Amount |  |

| Grand Total Amount |  |
| --- | --- |
| Delivery Offered | [Insert number of days/weeks/Months] |
| Warranty Provided | [(Insert number of days/weeks/Months] |

- [Insert Value in figures in EC$]

- [Insert Value in words in EC$]

Date:

Signature of Supplier

Insert Name of Supplier

Notes:

- The Supplier is required to mention make or model, as applicable, of the goods to be supplied and must attach the appropriate original printed literature / brochures for the various items listed.

- If there is an inconsistency between written words and figures, the words will prevail.

Section 6: Quotation Form

On letterhead of Tenderer

Date: [insert date]

Request for Quotations No.: [insert number]

| To: [Insert Name and address of the Procuring Entity] |  |
| --- | --- |

Having examined the Request for Quotations, the receipt of which is here by duly acknowledged, we, the undersigned, offer to supply [Insert name of contract] in conformity with the said Request for Quotation in total Price:

- [Insert Value in figures in EC$]

- [Insert Value in words in EC$]

We undertake, if our Quotation is accepted, to deliver the goods in [days/weeks / months] from the date of the signed contract, in accordance with the delivery time specified in clause 7 of the Contract.

We are not participating as Tenderers in more than one Quotation. Our Quotation shall be valid for [number days] after the deadline for submission of Quotations.

We declare that ourselves and any Suppliers for any part of the Contract have not been declared ineligible by the Government for offences or practices prohibited by the Act and furthermore pledge not to indulge such practices in the duration of the Contract.

We understand that your written Notification of Award shall constitute the acceptance of our Quotation and shall become a binding contract between us, until a formal contract is prepared and executed.

We understand that you are not bound to accept the lowest or any Quotation that you may receive.

Signature of Supplier

Name of Supplier

Section 7: Tenderer Information Form

Tenderer Information Form

[The Tenderer shall fill in this Form in accordance with the instructions indicated below. No alterations to its format shall be permitted and no substitutions shall be accepted.]

Date: [insert date (as day, month and year) of Submission of the Quotation]

RFQ No.: [insert number of tendering process]

Page ________ of_ ______ pages

1. Tenderer’s Name [insert Tenderer’s legal name]

2. Tenderer’s year of registration: [insert Tenderer’s year of registration]

3. Tenderer’s Address: [insert Tenderer’s legal address]

4. Other requirements: [insert as appropriate] [For example: business licences; copy of tax payment certificates etc…]

Section 8: Notification of Intention to Award a Contract

[Name of Procuring Entity]

[Name of Contract]

This Notification is sent by: [email] on [insert date]

# Notification of Intention to Award a Contract

[Send this Notification to the Tenderer’s Authorized Representative named in the Tenderer Information Form. This Notification of Intention to Award shall be sent simultaneously to each Tenderer that submitted a Quotation – delete this instruction].

For the attention of

Name: [insert Authorised Representative’s name]

Address: [insert Authorised Representative’s Address]

Email Address: [insert Authorised Representative’s email address]

Procuring Entity: [insert the name of the Procuring Entity]

Contract title: [insert the name of the contract]

This Notification of Intention to award a Contract (Notification) notifies you of our decision to award the above Contract. You may submit a procurement-related Complaint in relation to the decision to award the Contract.

- The successful Tenderer

| Name: | [insert name of successful Tenderer] |
| --- | --- |
| Address: | [insert address of the successful Tenderer] |
| Contract price: | [insert contract price of the successful Quotation] |

- Other Tenderers [INSTRUCTIONS: insert names of all Tenderers that submitted a Quotation. If the price was evaluated include the evaluated price as well as the Quotation price]

| Name of Tenderer | Tender price | Evaluated Tender price (if applicable) |
| --- | --- | --- |
| [insert name] | [insert Tender price] | [insert evaluated price] |
| [insert name] | [insert Tender price] | [insert evaluated price] |
| [insert name] | [insert Tender price] | [insert evaluated price] |

- How to make a complaint

At this point in the procurement process, you may submit a procurement-related Complaint challenging the decision to exclude you from the award of a contract. Your complaint must be submitted before the signature of the contract. Further information: refer to Part VII of the Act. In making a complaint, provide the Contract name, reference number, name of the Tenderer and contact details.

If you have any questions regarding this Notification, please do not hesitate to contact us.

On behalf of the Procuring Entity:

Signature: 	______________________________________________

Name:	______________________________________________

Title/position:	______________________________________________

Telephone:	______________________________________________

Email:	_____________________________________________

Section 9: Notification to Award a Contract

[Name of Procuring Entity]

[Name of Contract]

This Notification is sent by: [email] on [insert date]

Notification to Award a Contract

Notification to Award a Contract

Contract No. [insert RFQ reference number]

This is to notify you that your Quotation dated [insert date] for execution of the [insert name of the contract and identification number] for the Accepted Contract Amount of [insert amount in numbers and words and name of currency], as corrected and modified in accordance with the Instructions to Tenderers is hereby accepted by our Agency.

Please sign, date and return the Contract within [insert number] days of receipt of the same.

| Authorised Signature: |  |
| --- | --- |
| Name: |  |
| Title/position: |  |
| Name of Agency: |  |
| Telephone: |  |
| Email: |  |

Attachment: Contract

Section 10: Contract for the Supply of Goods

Contract Agreement

THIS AGREEMENT is made the [day] day of [month] 2025 between Central Procurement Unit, Galleria Mall, grand-anse, St. George’s (hereinafter called “the Procuring Entity”) and [insert name and address of Supplier] (hereinafter called “the Supplier”).

WHEREAS the Procuring Entity invited for quotation for the procurement of 2 EV buses and has accepted a Quotation by the Supplier for the supply of those goods in the sum of EC$ [insert Contract Price in figures and in words] (hereinafter called “the Contract Price”).

NOW THIS CONTRACT IS AGREED AS FOLLOWS:

      - The documents forming the Contract shall be interpreted in the following order of priority:

- the signed Contract Agreement

- Schedule of Items and Prices

- Technical Specification of the Goods Required

- List of Goods and Delivery Schedule.

    - The original printed literature / brochures for the various items listed

- In consideration of the payments to be made by the Procuring Entity to the Supplier as hereinafter mentioned, the Supplier hereby covenants with the Procuring Entity to provide the goods and to remedy any defects therein in conformity in all respects with the provisions of the Contract.

- The Procuring Entity hereby covenants to pay the Supplier in consideration of the provision of the goods and the remedying of defects therein, the Contract Price or such other sum as may become payable under the provisions of the Contract at the time and in the manner prescribed by the Contract.

- This Contract shall be amended only by written agreement between the Procuring Entity and the Supplier.

- The Supplier warrants that it has complied with and will continue to comply with the integrity provisions of section 115 of the Public Procurement and Asset Disposal Act, No. 19 of 2015.

- Price adjustment and variation are not applicable to this contract.

- All goods shall be delivered to the Procuring Entity within [insert number of [days/weeks/months] from date of contract signature.

- The Supplier warrants that goods are contained or packaged adequately to protect the goods from loss or damage when transporting to the final destination of delivery.

- Within 3 days of delivery, the Procuring Entity shall inspect the goods delivered to ensure compliance with the requirements of the contract. If no fault is found, a certificate of acceptance shall be issued to the Supplier.

- Payment to the Supplier shall be made in full in EC$ against the Supplier’s invoice by transfer to the account of Supplier or by appropriate method of payment specified in the Supplier’s invoice following delivery of goods to the final destination and within [insert number of days/weeks/months] of the certificate of acceptance. Payment of VAT/Taxes and other impositions shown in the Supplier’s invoice shall be made in accordance with the regulations applicable at the time of settlement of the invoice.

- If the Supplier delivers the specified goods later than the delivery period specified in clause 7, the Procuring Entity has the right to reduce the payment to the supplier by  0.05% of the total price of contract for each day of delay beyond the delivery time. The deduction is up to a maximum of the 10%, at which point the Procuring Entity may terminate the contract.

- The Supplier warrants the use by the Procuring Entity of the goods sold under this contract does not infringe any patent, design, trade name or trademark. In addition, the supplier shall, pursuant to this warranty, indemnify, defend and hold the Procuring Entity harmless from any action or claim pertaining to the alleged infringement of patent, design, trade name or trademark arising in connection with the goods sold under this contract.

- The Supplier shall provide the warranty of [insert number months] and if any faults are detected within the warranty period in the supplied/installed goods, the Supplier shall be bound to rectify the fault or replace the goods within [insert number] days.

- The Procuring Entity may, by written notice sent to the Supplier, terminate the Contract in whole or in part at any time for its convenience:

    - if the Supplier fails to deliver any or all the goods within the time period(s) specified in this contract, or

    - if the Supplier fails to perform any other obligation(s) under this contract, or

    - if the Supplier, in either of the above circumstances does not cure its failure within a period of (3) three calendar days after receipt of a notice of default from the Procuring Entity specifying the nature of the default(s), or

    - if the Supplier, in the judgment of the Procuring Entity, has engaged in any corrupt or fraudulent practices in competing for or executing the tasks under this contract.

- If any dispute arises between the two parties relating to any aspects of this Contract, the parties shall first attempt to settle the dispute through mutual and amicable consultation. If, after fourteen (14) days, the parties have failed to resolve their dispute or difference by such mutual consultation, then either the Procuring Entity or the Supplier may give notice to the other party of its intention to commence arbitration in accordance with the laws of Grenada.

------------

IN WITNESS whereof the parties hereto have caused this Agreement to be executed in accordance with the laws of Grenada on the day, month and year first written above.

For and on behalf of the Procuring Entity:

Signed:_______________________

Full name: Mr. Mike Sylvester

Entity: Ministry of Finance

In the capacity of: Permanent Secretary

In the presence of   _________________________

For and on behalf of the Supplier:   _________________________

Signed: [insert signature of authorised representative(s) of the Supplier]

Full name: [name of person signing]

In the capacity of: [insert title or other appropriate designation]

In the presence of [insert identification official of witness]